CSUF SSP Participants Worksheet

| Name: | Date: |
| --- | --- |
| Module: 1 | Part: C |

Professor Comments:

# 1 - Debugging Activity: Fix the Flip-Flop

Debugging Activity: Fix the Flip-Flop

A flip-flop circuit has been built using logic gates. However, something went wrong. Every gate in the circuit was accidentally changed to an OR gate, so the circuit no longer works correctly. Your job is to debug the circuit and restore correct behavior.

https://wokwi.com/projects/459586165220865025

Goal

Using your knowledge of logic gates and truth tables, determine which type of logic gate(s) should replace the OR gates to make the flip-flop work correctly?

Expected Behavior

| D (Data) | E (Enable) | Q (Output) | !Q (Inverted Output) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 1 |
| 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 0 |

Success Criteria

- The simulation matches the truth table for all input combinations

- Q and !Q are always complementary

- The circuit behaves like a proper D flip-flop with enable

Reflection Questions

- What type of logic gate did you replace the OR gates with, and why did that choice fix the circuit?

- How does the Enable (E) signal affect the behavior of the flip-flop?

- Why must Q and !Q always be complements of each other?

- What was the main problem with using only OR gates in this circuit?

- If you were designing this circuit from scratch, how would you verify that it works correctly before building it?

# 2 - Short Technical Reflection

Why can’t all systems be purely digital? (5 - 8 sentences)

Source: Tiny Tapeout