Undergraduate SSP Participants Project

| Name: | Date: |
| --- | --- |
| Module: 4 | Part: A |

Professor Comments:

# Module 4 Project: ALU Design and Testing

## Overview

In this project, you will:

- Design a simple ALU in Verilog

- Implement multiple operations

- Write a testbench to verify correctness

- Use simulation as a testing tool

## Learning Goals

- Translate operations into hardware (Verilog)

- Build combinational logic using always @(*)

- Develop a testing mindset

- Use testbenches as automated tests

# ALU Implementation

## Step 1: Start with the Skeleton

module alu (

input  wire [7:0] A,

input  wire [7:0] B,

input  wire [1:0] opcode,

output reg  [7:0] result

);

always @(*) begin

case (opcode)

// TODO: implement operations

default: result = 0;

endcase

end

endmodule

## Step 2: Implement Operations (Incrementally)

Implement the following:

| Opcode | Operation |
| --- | --- |
| 00 | A + B (ADD) |
| 01 | A - B (SUB) |
| 10 | A & B (AND) |
| 11 | A ¦ B (OR) |

Requirement:

- Add one operation at a time

- After each addition, simulate and verify

## Step 3: Check Your Work

Before moving on, verify:

- Does each opcode produce the correct result?

- Does the output update when inputs change?

- Are there any unexpected values?

Common Mistakes to Watch For

- Missing case entries

- Using ^ instead of |

- Forgetting default

- Syntax errors (missing ;)

# Testbench and Simulation

## Step 4: Create a Testbench

Start with this template:

module alu_tb;

reg  [7:0] A;

reg  [7:0] B;

reg  [1:0] opcode;

wire [7:0] result;

alu uut (

.A(A),

.B(B),

.opcode(opcode),

.result(result)

);

initial begin

// TODO: write test cases

end

endmodule

## Step 5: Write Test Cases

You must include:

1. Basic Tests (one per operation)

Example: A = 10; B = 3; opcode = 2'b00; #10; // ADD

2. Edge Cases

Include at least one:

- A = 0, B = 0

- Large values (e.g., 255 + 1)

- A < B for subtraction

3. Multiple Tests

- At least 5 total test cases

## Step 6: Run Simulation

Use: Vivado Simulator

Process:

- Compile

- Run simulation

- Observe outputs

## Step 7: Waveform Analysis

Check:

- Does the result match expected values?

- Does output change immediately when inputs change?

- Do opcode changes produce correct operation?

## Step 8: Debug if Needed

If something is wrong:

- Re-check your case statements

- Verify opcode mapping

- Compare expected vs actual values

## Deliverables:

- alu.v - Your ALU implementation

- alu_tb.v - Your testbench

- Short Answer (2–3 sentences each)

  - What bug did you encounter (if any)?

  - How did simulation help you find it?