Undergraduate SSP Participants Project

| Name: | Date: |
| --- | --- |
| Module: 4 | Part: B |

Professor Comments:

Debugging and Verification Lab

Goal

- Use simulation and waveforms to find and fix bugs

- Think like a hardware tester, not just a designer

Part 1: Given a Broken ALU

You are given the following ALU implementation:

module alu_buggy (

input  wire [7:0] A,

input  wire [7:0] B,

input  wire [1:0] opcode,

output reg  [7:0] result

);

always @(*) begin

case (opcode)

2'b00: result = A + B;   // ADD

2'b01: result = A - B;   // SUB

2'b10: result = A & B;   // AND

2'b11: result = A ^ B;   // BUG?

endcase

end

endmodule

Part 2: Use This Testbench

module alu_buggy_tb;

reg  [7:0] A;

reg  [7:0] B;

reg  [1:0] opcode;

wire [7:0] result;

alu_buggy uut (

.A(A),

.B(B),

.opcode(opcode),

.result(result)

);

initial begin

A = 8'd10; B = 8'd3; opcode = 2'b00; #10;

A = 8'd10; B = 8'd3; opcode = 2'b01; #10;

A = 8'd10; B = 8'd3; opcode = 2'b10; #10;

A = 8'd10; B = 8'd3; opcode = 2'b11; #10;

$finish;

end

endmodule

Part 3: Your Tasks

Task 1: Predict Before Simulating

Fill this table before running the simulation:

| Opcode | Operation | Expected Result |
| --- | --- | --- |
| 00 | ADD |  |
| 01 | SUB |  |
| 10 | AND |  |
| 11 | OR |  |

Task 2: Run Simulation

- Run the testbench

- Observe the output (console or waveform)

Task 3: Identify the Bug

- Which opcode produces the wrong result?

- What result did you get vs what you expected?

Task 4: Debug Using Waveforms

Look at:

- A, B

- opcode

- result

Answer:

- Does the wrong value appear immediately after opcode change?

- Is the issue timing-related or logic-related?

Task 5: Fix the ALU

- Identify the incorrect line

- Correct it

- Re-run simulation

Task 6: Verify the Fix

- Do all outputs now match expectations?

- Did your waveform confirm the fix?

Part 4: Reflection (Testing Mindset)

Answer briefly:

- What was the bug?

- How did simulation help you find it?

- Why is guessing not a good debugging strategy in hardware?