1 - Map to Chip Types

Students classify system blocks as Digital, Analog, or RF and justify.

Sample Answers

| Block | Type | Why? |
| --- | --- | --- |
| Microcontroller / CPU | Digital | Performs logic, executes instructions using binary signals |
| Temperature sensor (analog output) | Analog | Produces continuous voltage proportional to temperature |
| ADC (Analog-to-Digital Converter) | Mixed (Analog + Digital) | Converts continuous signal into digital data |
| Amplifier | Analog | Conditions small signals into usable range |
| Wi-Fi / Bluetooth module | RF | Transmits/receives wireless signals |
| GPIO / Control logic | Digital | Sends discrete on/off control signals |

TA Guidance

- Look for correct classification based on signal type

- Justifications should reference:

  - Digital → discrete/binary logic

  - Analog → continuous signals

  - RF → wireless communication

- Mixed-signal answers (e.g., ADC) are a good sign of deeper understanding

2 - System Comparison

Sample Answers

Which parts are always digital?

- Control logic, microcontrollers, processors

- Decision-making and program execution blocks

Where is analog unavoidable?

- At the interface with the real world (sensors, microphones, cameras)

- Signal conditioning (amplifiers, filters)

When is RF required?

- When wireless communication is needed (Wi-Fi, Bluetooth, cellular)

- For transmitting data over the air

TA Guidance

- Students should recognize digital = computation/control

- Analog is tied to physical signals

- RF is specifically for wireless communication, not just “fast signals”

3 - System Designs

Sample Answer: Temperature-Controlled Fan

Inputs:

- Temperature sensor (analog voltage or digital sensor reading)

Outputs:

- Fan on/off signal (or PWM for speed control)

Processing Logic:

- Read temperature value

- Compare to threshold

- If temperature > threshold → turn fan ON

- Else → turn fan OFF

Sample Block Diagram

[ Temperature Sensor ] → [ ADC (if needed) ] → [ Microcontroller / Comparator ] → [ Fan Driver ] → [ Fan ]

TA Guidance

- Inputs should clearly reflect temperature sensing

- Processing must include a comparison (threshold logic)

- Outputs should connect logically to a fan actuator

- Diagram should show signal flow and meaningful intermediate blocks

What to Look For

- Correct mapping of signal types to chip domains (digital/analog/RF)

- Understanding that real-world signals start analog

- Recognition that digital systems make decisions

- Ability to design a simple control system with clear logic flow

- Block diagrams that go beyond a single “black box” and show structure