1 - Debugging Activity: Fix the Flip-Flop

Correct Fix

The OR gates should be replaced with NAND gates (or equivalently NOR gates, depending on implementation style).

Most standard gated D latch / flip-flop designs use:

- NAND-based SR latch (active-low) or

- NOR-based SR latch (active-high)

Given the behavior described, NAND gates are the most likely intended fix.

Why This Fix Works

- Flip-flops require feedback and bistability

- OR gates cannot create proper memory behavior because:

  - They do not enforce complementary outputs

  - They cannot properly “hold” state in feedback loops

- NAND/NOR gates enable:

  - Stable storage of 1 bit

  - Proper inversion relationship between Q and !Q

  - Controlled updating via Enable (E)

Expected Behavior Explanation

- When E = 0 → latch holds previous value

- When E = 1 → Q follows D

- Q and !Q must always be opposites

Reflection Question Sample Answers

What type of logic gate did you replace the OR gates with, and why did that choice fix the circuit?

- Replaced with NAND gates because they allow feedback that creates stable memory and enforce complementary outputs.

How does the Enable (E) signal affect the behavior of the flip-flop?

- Enable controls whether the circuit updates or holds its value.

- When enabled, input D is passed to Q; otherwise, the previous state is retained.

Why must Q and !Q always be complements of each other?

- They represent the stored bit and its inverse.

- If they are not complements, the circuit is in an invalid or unstable state.

What was the main problem with using only OR gates in this circuit?

- OR gates cannot create proper bistable memory behavior.

- They do not enforce inversion or stable feedback needed for a latch.

If you were designing this circuit from scratch, how would you verify that it works correctly before building it?

- Simulate all input combinations

- Check truth table behavior

- Verify Q and !Q are always complementary

- Test edge cases (enable on/off transitions)

TA Guidance

- Look for recognition of bistability and feedback

- Students should identify that OR gates fundamentally cannot implement a latch

- Strong answers mention why NAND/NOR work, not just that they do

- References to truth tables or simulation indicate deeper understanding

2 - Short Technical Reflection

Sample Answer (5–8 sentences)

Not all systems can be purely digital because the real world is inherently analog. Physical quantities like temperature, sound, and light vary continuously, so they must first be sensed as analog signals. These signals often need amplification or filtering before they can be converted into digital form. Additionally, actuators like speakers or motors require analog signals to function properly. Digital systems are excellent for computation and control, but they rely on analog interfaces to interact with the environment. This is why most real systems are mixed-signal, combining both analog and digital components. Without analog components, a system would not be able to sense or affect the physical world.

TA Guidance

- Students should clearly articulate:

  - Real-world signals are analog

  - Need for sensing and actuation

  - Role of mixed-signal systems

- Strong answers connect to earlier concepts:

  - ADCs, sensors, signal conditioning

What to Look For

- Understanding of why certain logic gates enable memory (bistability)

- Ability to connect truth tables → circuit behavior

- Recognition that Enable controls state updates

- Clear reasoning about analog vs. digital limitations

- Evidence of systems-level thinking (not just memorization)