1 - CMOS Fundamentals

Sample Answers

What does CMOS stand for?

Complementary Metal-Oxide-Semiconductor

What is the role of a transistor in a circuit?

A transistor acts as an electronic switch or amplifier, controlling the flow of current in a circuit based on an input signal.

What are the two types of transistors used in CMOS?

- NMOS

- PMOS

Describe what each transistor type does (in your own words):

- NMOS: Turns ON when the input is high (1) and pulls the output toward ground (0)

- PMOS: Turns ON when the input is low (0) and pulls the output toward Vdd (1)

TA Guidance

- Students should understand:

  - CMOS = complementary pairing of NMOS and PMOS

  - Transistors behave like switches

  - NMOS and PMOS have opposite control behavior

2 - CMOS Inverter Reasoning

Correct Table

| Input | NMOS State | PMOS State | Output |
| --- | --- | --- | --- |
| 0 | OFF | ON | 1 |
| 1 | ON | OFF | 0 |

TA Guidance

- Key concept: only one transistor conducts at a time (ideally)

- Students should connect:

  - NMOS → pulls down

  - PMOS → pulls up

- Output must be the inverse of input

3 - Why CMOS Is Used Everywhere

Sample Answers

- CMOS has very low power consumption, especially when not switching

- CMOS provides good noise margins and reliability

- It scales well with technology (small feature sizes)

- High integration density (millions/billions of transistors)

(Any two valid reasons)

TA Guidance

- Strong answers mention:

  - Low static power

  - Scalability

- Avoid vague answers like “because it is better”

4 - Concept Check

Correct Answers

- CMOS circuits use only one type of transistor → FALSE

- CMOS helps reduce power consumption → TRUE

- CMOS is used in modern CPUs → TRUE

- Transistors act like switches → TRUE

TA Guidance

- Look for consistency with Part 1 concepts

- Misconceptions often show up here (especially the first question)

What to Look For

- Understanding of complementary transistor behavior (NMOS vs PMOS)

- Ability to reason through a CMOS inverter

- Recognition of why CMOS dominates (power + scalability)

- Correct interpretation of basic digital electronics concepts