1 - Design Flow Ordering

Correct Order

- Design

- Synthesis

- Layout

- Fabrication

- Testing

TA Guidance

- Students should understand the flow moves from abstract → physical → verification

- Fabrication must come after layout, not before

2 - What Happens at Each Step?

Sample Answers

Design:

- Define system behavior and write logic (e.g., HDL or block diagrams)

Synthesis:

- Convert design into a gate-level implementation using logic gates

Layout:

- Translate gates into physical geometry (placement and routing on silicon)

Fabrication:

- Physically manufacture the chip in a semiconductor process

Testing:

- Verify the fabricated chip works correctly and meets specifications

TA Guidance

- Look for increasing concreteness across steps

- Strong answers connect:

  - Design → logic

  - Synthesis → gates

  - Layout → physical structure

3 - Why the Flow Matters

Sample Answers

Why are mistakes late in the process expensive?

- Because fabrication is costly and time-consuming, so fixing errors requires redesign and rebuilding the chip

Why can’t we skip steps?

- Each step builds on the previous one, so skipping steps would result in an incomplete or incorrect design

TA Guidance

- Students should recognize:

  - Cost of fabrication

  - Dependency between steps

4 - Apply the Flow

Sample Answers

Inputs:

- Motion sensor signal

Outputs:

- Light ON/OFF

Processing Logic (if–then):

- If motion detected → turn light ON

What would you do during design?

- Define system behavior and logic (motion detection rule)

What would synthesis produce?

- A gate-level circuit implementing the logic

What happens in layout?

- Gates are arranged and connected physically on silicon

TA Guidance

- Inputs/outputs must match system description

- Processing should reflect clear if–then logic

- Flow mapping should align with Part 2 concepts

5 - Reflection

Sample Answer (3–5 sentences)

Layout is often the most complex step because it requires translating logical designs into physical geometry while meeting strict constraints. Issues like timing, area, and routing must all be considered. Small mistakes can cause the chip to fail even if the logic is correct. It also requires specialized tools and expertise.

(Other answers acceptable: fabrication or synthesis, with justification)

TA Guidance

- Look for:

  - Justification tied to complexity or constraints

  - Awareness of real-world challenges (cost, tools, precision)

What to Look For

- Understanding of the full chip design flow

- Ability to connect abstract logic → physical implementation

- Recognition of cost and risk in later stages

- Consistency between system description and flow mapping

- Thoughtful reflection showing awareness of engineering tradeoffs