1 - Datapath Definition

Sample Answer

A datapath is the part of the CPU that moves and processes data. It consists of components like registers, ALUs, and buses that work together to execute instructions. The datapath performs the actual operations specified by instructions.

TA Guidance

- Look for:

  - Movement of data

  - Processing of data

  - Mention of hardware components (registers, ALU, etc.)

2 - Core Components

Sample Answers

- Registers – Store data temporarily inside the CPU

- ALU (Arithmetic Logic Unit) – Performs arithmetic and logical operations

- Multiplexers (MUXes) – Select between multiple inputs

- Buses – Carry data between components

- Program Counter (PC) – Holds address of next instruction

- Instruction Register (IR) – Holds current instruction

TA Guidance

- Students should identify multiple components, not just one

- Descriptions should reflect function, not just naming

3 - Datapath Operation (Step-by-Step)

Instruction: ADD R3, R1, R2

Sample Steps

- Read values from registers R1 and R2

- Send values through datapath to ALU inputs

- ALU performs addition

- Result is routed back through datapath

- Store result in destination register R3

TA Guidance

- Look for:

  - Data movement

  - ALU usage

  - Write-back step

- Strong answers include flow through datapath elements

4 - Hardware Mapping

Source registers:

- R1, R2

Destination register:

- R3

Functional unit used:

- ALU

TA Guidance

- Must distinguish sources vs destination

- ALU should be identified clearly

5 - Control Signals

Sample Answers

- RegRead – Enables reading from source registers

- ALUOp – Specifies operation (addition)

- RegWrite – Enables writing result to destination register

(Other valid answers: MUX select signals, write enable, etc.)

TA Guidance

- Students should connect control signals to:

  - enabling data movement

  - selecting operations

- Strong answers include both what and why

6 - Program Counter Behavior

Sample Answers

Normal execution:

- PC increments to the next sequential instruction

Branch/jump instruction:

- PC is updated to a new address based on the branch or jump target

TA Guidance

- Look for:

  - Sequential increment

  - Non-sequential update for control flow

- Avoid vague answers like “PC changes”

What to Look For

- Understanding of datapath as data movement + computation system

- Ability to trace instruction execution through hardware

- Recognition of roles of registers, ALU, and control signals

- Clear mapping between instruction → hardware resources

- Awareness of control flow via the Program Counter