1 - Instruction Format Purpose

Sample Answer

An instruction format defines how an instruction is structured in binary, including fields for operation type and operands. It is important because it allows the CPU to correctly decode and execute instructions. A consistent format ensures efficient hardware implementation and instruction parsing.

TA Guidance

- Look for:

  - Structure/layout of instruction

  - Role in decoding/execution

- Strong answers mention hardware efficiency or consistency

2 - Identify Fields

Correct Explanations

opcode: Specifies the operation to perform (e.g., ADD, SUB)

rs: First source register

rt: Second source register

rd: Destination register (where result is stored)

funct: Further specifies the exact operation (used with opcode in some architectures)

TA Guidance

- Students should distinguish:

  - operation vs operands

- Common mistake: confusing rd with source registers

3 - Compare Instruction Types

Sample Answers

| Type | Description | Example |
| --- | --- | --- |
| R-type | Register-to-register operations | ADD R1, R2, R3 |
| I-type | Includes immediate value or memory access | ADDI R1, R2, #5 |
| J-type | Used for jumps to a target address | JMP address |

TA Guidance

- Look for:

  - R-type → register operations

  - I-type → immediate or memory

  - J-type → control flow

4 - Indexed Addressing (Base + Offset)

Correct Answers

Base register: R2

Offset: 8

Effective Address (EA) calculation: EA = R2 + 8

TA Guidance

- Students should clearly identify:

  - Base vs offset

  - Addition for address calculation

5 - Datapath Interaction

Sample Answer

The datapath reads the value from the base register (R2) and sends it to the ALU. The offset value (8) is also provided to the ALU. The ALU adds these two values to compute the effective address, which is then used to access memory.

TA Guidance

- Look for:

  - Register read

  - ALU addition

  - Address used for memory access

6 - Encoding Challenge

Solution

opcode = 6 bits

rs = 5 bits

rt = 5 bits

rd = 5 bits

additional fields = 11 bits

Total = 6 + 5 + 5 + 5 + 11 = 32 bits

TA Guidance

- Students should:

  - Sum all fields correctly

  - Show work

- Reinforces standard 32-bit instruction format

What to Look For

- Understanding of instruction structure and fields

- Ability to distinguish between instruction types (R/I/J)

- Correct reasoning about address calculation (base + offset)

- Connection between instruction fields and datapath behavior

- Comfort with bit-level encoding and instruction size