1 - Big Picture

Sample Answer

The control unit directs the operation of the processor by generating control signals based on the instruction. The datapath carries out the actual data movement and computation using components like registers and the ALU. Together, they enable instructions to be executed correctly.

TA Guidance

- Look for:

  - Control unit = decision/control signals

  - Datapath = execution/data movement

- Strong answers emphasize coordination between the two

2 - Instruction → Hardware

Instruction: ADD R1, R2, R3

Control unit does:

- Decodes instruction

- Generates control signals (select registers, ALU operation, write enable)

Datapath does:

- Reads values from R2 and R3

- Sends them to ALU

- Performs addition

- Writes result to R1

TA Guidance

- Students should separate:

  - “what controls” vs “what executes”

- Look for correct data flow

3 - Control Signal Reasoning

Correct Answers

ALU operation: ADD

Register write enable: Enabled (1)

Memory access: None (no read/write to memory)

TA Guidance

- Students should recognize:

  - ADD is register-only operation

  - No memory involved

- Common mistake: saying memory is used

4 - System Components

Sample Answers

- Control unit

- Datapath

- Registers

- ALU

- Memory

- Buses/interconnect

TA Guidance

- Look for multiple components

- Should include both control + datapath elements

5 - Design Tradeoffs

Sample Answers

Performance: How fast the processor executes instructions (e.g., speed, throughput)

Efficiency: How well the processor uses resources such as power, area, or energy

TA Guidance

- Students should distinguish:

  - Performance = speed

  - Efficiency = resource usage

- Strong answers may mention power or energy

6 - Putting It All Together

Instruction: LDR R1, [R2]

What the control unit does:

- Decodes instruction

- Enables memory read

- Selects correct registers and data paths

What the datapath does:

- Reads address from R2

- Sends address to memory

- Retrieves data from memory

- Writes data into R1

Hardware components involved:

- Registers (R1, R2)

- ALU (for address handling if needed)

- Memory unit

- Buses

- Control unit

TA Guidance

- Look for full system-level thinking:

  - Address → memory → data → register

- Strong answers mention data movement across components

7 - Reflection (Higher-Level Thinking)

Sample Answer (3–5 sentences)

Thinking of instructions as hardware actions helps us understand how software actually runs on a computer. It shows how each instruction translates into physical operations inside the CPU. This perspective helps with debugging, optimization, and system design. It also connects programming concepts to real hardware behavior.

TA Guidance

- Students should connect:

  - Software → hardware mapping

  - Benefits (debugging, optimization, understanding)

What to Look For

- Clear distinction between control unit and datapath roles

- Ability to map instructions to control signals and hardware actions

- Understanding of register vs memory operations

- Awareness of system-level components and interactions

- Higher-level thinking connecting code to physical execution