Undergraduate SSP Participants Worksheet

| Name: | Date: |
| --- | --- |
| Module: 4 | Part: A |

Professor Comments:

# 1 - Predict the Output

For each snippet:

- Determine the output y

- Assume inputs are given values

Problem A:

assign y = a & b;

If:

- a = 1

- b = 0

What is y?

Problem B:

assign y = a | b;

If:

- a = 0

- b = 0

What is y?

Problem C:

assign y = a ^ b;

If:

- a = 1

- b = 1

What is y?

# 2 - What Operation is This?

Identify what the circuit is doing.

Problem A:

assign y = A + B;

- What operation is this?

- What kind of hardware implements it?

Problem B:

assign y = A & B;

- What operation?

- What hardware?

# 3 - Find the Bug (Syntax / Logic)

Each snippet has a mistake.

Problem A:

module test(input a, input b, output y)

assign y = a & b;

endmodule

- What is wrong?

Problem B:

module test(

input a,

input b,

output reg y

);

assign y = a | b;

endmodule

- What is wrong?

Problem C:

always @(*) begin

y = a + b

end

- What is wrong?